What’s New

in CAMWorks

HCL Technologies Ltd. makes no warranties, either expressed or implied with respect to this  document. HCL reserves the right to revise and improve products as it sees fit, and to revise the  specifications and information contained herein without prior notice. Due to continuing product  development, the specifications and capabilities described in this document are subject to change  without notice.

Copyright © 2025 HCL Technologies Ltd. All Rights Reserved.

August 2025

Table of Contents

Table of Contents

(Click a link below or use the bookmarks on the left.)

What’s New in CAMWorks 2026 BETA2 3 Supported Platforms 3 Resolved CPRs Document 3 Discontinuation of MS Access Based TechDB 3 Support for Swiss Lathes in CAMWorks...................................................................4 Support for Swiss Turn Machines (Multi-Function Lathes) in CAMWorks 4 Machine Components & Definition, Tool Cribs, and Setup.....................................6 Forms to create, edit and save Swiss Lathe/Multi-Function Lathe Machine Definitions within TechDB 6 User-assigned names for Turrets / Tool posts associated with Mill-Turn / Swiss Lathes / Multi-Function Lathes 8 Machine Groups Option to sort and classify Mill-Turn/Multi-Function Lathe Machines 13 Toolcrib Groups Option to sort and classify Mill-Turn Tool Cribs 15 Collets as Work Holding Device for Turn and Mill-Turn Machines 18 Option to Define Collet Housing Parameters 21 Activating Sliding Head Option for a Selected Machine 23 Guide Bushings when Machining with Mill-Turn/Multi-Function Lathe Machines 25 ’Initial collet rechuck Z’ to indicate distance between Collet and Part Origin 28 Support Turn And Mill-Turn Fixtures In Z Positive Space 30 Option to Enable/Disable the Creation of Duplicate Station Numbers for Stations in Tool Cribs 34 Tools ...........................................................................................................................36 Cutting Parameters for Turn Inserts Based on Stock Material 36 Option to Assign Tool Offset, Compensation IDs, and Tool Shift Parameters 39 Spindle Association with Tool Station to Define Work Area 41 Option to Create Empty Tool Stations in Tool Cribs and associated Functionalities 44 Orientation for Mill Tools in Mill-Turn Tool Cribs 50 New Tool Changer Option ‘Slide’ available For All Machine Configurations within TechDB 52 Operations and Toolpaths ........................................................................................53 Segmentation Manager for Turn Operations 53 Support for Bar Break Chamfering of the Stock in Turn Toolpaths 57 Controlling Z Axis Movement for Toolpaths of Parts programmed for Swiss-Turn Machines 60 Indicating whether a Tool Post/ Turret/ Gang Slide of Swiss-Turn Machines is capable of movement in Z axis 64 Associating Sub Spindle operations with other Operations to ensure Auto-Synchronization 66 Chip Breaking Functionality for Turn Operations 67 Simulation and Visualization....................................................................................69 Display of Machine Kinematics during Toolpath Simulation 69 Machine Awareness Functionality Support For CNC Machines (Including Swiss Lathes) 71 Support for Swiss Turn Machines in CAMWorks Virtual Machine Environment 75 Expanded Sync Manager View for CAMWorks Sync Manager 77 Providing Visual Feedback on Start Location of Wait Codes within CAMWorks Sync Manager 79 Option to Create User Defined Folders under CAMWorks Trees 82 Post Processors to generate NC code for Toolpaths Generated on Swiss Turn Machines 85 CAMWorks Tools Tree view under Tool Cribs Form of Technology Database 86

2 | P a g e

What’s New in CAMWorks 2026 BETA2

What’s New in CAMWorks 2026 BETA2

Supported Platforms

| Supported Platforms for 64-bit |  |
| --- | --- |
| Solid Modeler: | The 64-bit version of: - SOLIDWORKS 2025 - CAMWorks Solids 2025 Note: SOLIDWORKS 2026 and CAMWorks Solids 2026 will be supported once released. |
| Operating System: | 64-bit version of: - Windows 11 - Windows 10 [*Home Editions are not supported] Note: CAMWorks 2026 is supported only on 64-bit Operating systems. |

Resolved CPRs Document

| Purpose of Document: | The Resolved CPR (CAMWorks Problem Report) document has been updated to report the software errors that have been resolved in the current Service Pack (BETA). |
| --- | --- |
| Path to Document: | To view the document, select: C:\Program Files\CAMWorks2026x64\CAMWorks_VC143\Lang\English\CW2026BuildInfo.pdf |

Discontinuation of MS Access Based TechDB

The SQLite based TechDB is the default TechDB format linked to the CAMWorks application at the  time of installation/upgradation. In the CAMWorks versions prior to CAMWorks 2026 BETA, the other  database formats supported were the SQL Server based TechDB and MS Access based TechDB  formats.

From CAMWorks 2026 BETA version onwards, the only database format other than the default SQLite  based TechDB format will be the SQL Server based TechDB format. The MS Access based TechDB  can no longer be linked to CAMWorks 2026 BETA and later versions.

Once you upgrade your CAMWorks installation to 2026 BETA or any later version, only the SQLite based TechDB or SQL Server based TechDB can be assigned as the default TechDB linked to the  application. If the default TechDB previously linked to your CAMWorks installation was an MS Access based TechDB containing customized TechDB data, then you will need to import this data into the  SQLite/SQL Server based TechDB once you upgrade to a newer version of the CAMWorks application.

3 | P a g e

What’s New in CAMWorks 2026 BETA2  Support for Swiss Lathes in CAMWorks

Support for Swiss Turn Machines (Multi-Function Lathes) in CAMWorks

Swiss Turn Machine: A Modern Manufacturing Marvel In the world of modern manufacturing, the demand for compact, precise, and intricate parts continues to rise — especially in industries like medical devices, aerospace, electronics, and watchmaking. To meet these challenges, engineers and machinists have long turned to a special kind of turning center: the Swiss-Type Lathe, often simply known as the Swiss-Turn. Swiss Turn: A Machine Born for Precision The story of the Swiss-Turn begins in the Swiss watchmaking industry, where the need for finely detailed components led to the development of a revolutionary approach to turning. Unlike traditional lathes, where the workpiece (stock) stays fixed and tools move to cut it, the Swiss-Turn machine does the opposite: the bar stock itself moves longitudinally through a guide bushing. This setup brings the cutting action close to the support point, drastically reducing deflection, especially important for long, slender parts. This design unlocked a world of possibility: machinists could now produce extremely detailed components with higher accuracy, better surface finish, and improved efficiency. Over time, Swiss Turn machines evolved to support multi-axis operations, live tooling, and sub-spindle machining, making them incredibly versatile and capable of complete part machining in a single cycle. Advantages of Swiss Turn Machines • Precision: Capable of maintaining tighter tolerances • Speed: Multiple operations running simultaneously across turrets and spindles • Efficiency: Less need for secondary operations or setups • Durability: Lower tool wear due to reduced vibration and stable machining • Complexity Made Simple: Ideal for parts with intricate geometries and tight specs • Lesser Monitoring: Machines can operate and produce parts without any human supervision CAMWorks Workflow for Machining with Swiss Turn Machines : Tailored for Success Let’s follow the journey of a part as it moves through the CAMWorks Swiss-Turn process with the example given below: Step 1: The Design Consider that an engineer designs a complex medical implant. The part is slender, with multiple grooves, drilled holes, and fine details—perfect for machining using a Swiss Turn machine. Step 2: Machine Setup Inside CAMWorks, the user selects a pre-configured Swiss Turn machine with Main and Sub spindles and tool posts. The stock and work holding setups are defined. Step 3: Toolpath Generation Using its Feature-Based Machining functionality, CAMWorks identifies machining features—such as outer and inner turning diameter, grooves, cross holes, etc., and automatically generates the appropriate toolpaths. Each operation is assigned to the correct turret or spindle, and machining strategies are optimized. Step 4: Synchronization Magic CAMWorks’ Sync Manager functionality will let you coordinate every move between the spindles and turrets on a graphical timeline. Need to rough-turn on the Main spindle while drilling on the sub spindle? Just drag and drop a sync marker! Transferring the part to the sub-spindle is just a few clicks away.

4 | P a g e

What’s New in CAMWorks 2026 BETA2

Step 5: See It Before You Cut It The complete operation sequence is run through the machine simulation. You can view and verify while tools move in harmony and the part takes shape. You can check for collisions, optimize the cycle time, and make real-time adjustments. Step 6: From Code to Cutting Finally, CAMWorks will post-process the toolpaths into ready-to-run NC code, custom-tailored for Swiss-Turn machines. The part program will then be transferred to the shop floor for precision production. The fast cycle times and flexibility of Swiss Turn machines make them the standard in high precision manufacturing applications. They are recommended for small, long, slender, turned parts with complex geometry. Overall, you can trust CAMWorks to deliver on efficiency, precision, correctness, and control when it comes to Swiss Turn Machine Programming!

The following pages of this document cover the various functionalities and user interfaces  introduced/modified in CAMWorks to support the machining of parts using Swiss Turn machines.

5 | P a g e

What’s New in CAMWorks 2026 BETA2

Machine Components & Definition, Tool Cribs,  and Setup

Forms to create, edit and save Swiss Lathe/Multi-Function Lathe Machine Definitions within  TechDB

Purpose: Provision to view, create, edit, and save Swiss Lathe/Multi-Function Lathe Machine definitions within the Technology Database

Implementation: From CAMWorks 2026 version onwards, within the Technology Database, the Mill-Turn menu on the left has been renamed to Mill-Turn/Multi-Function Lathe menu. Swiss Lathes and Multi-Function Lathe machines can be viewed, defined, edited, and saved within the Machines form available under this menu. All sample Mill-Turn/Swiss Lathe/Multi-Function Lathe machines shipped along with CAMWorks are listed within the Machines form under this menu. To create the machine definition corresponding to the Swiss Lathes/Multi-Function Lathes used at your machining facility within the TechDB, follow the steps bellow: 1. Within the TechDB, click on the Mill-Turn/Multi-Function Lathe menu. 2. Under the Machines sub-menu with the displayed interface, select any Machine Definition and click on the Copy button at the top of the interface. 3. The newly created Machine Definition will be listed under the Machines sub-menu and will be auto selected. 4. Use the Machines form to edit the various parameters associated with the machine definition and save the changes by clicking on the Save button. This action will save the machine definition. Mill-Turn/Multi-Function Lathe>Machines Form of TechDB to view, create, edit, and save Swiss Lathe and Multi-Function Lathe Machine Definitions When programming a part or assembly using the CAMWorks application, this newly added machine will become available, along with other saved machines in the TechDB, for machine selection under

6 | P a g e

What’s New in CAMWorks 2026 BETA2

the Machine tab of the Machine dialog box. Mill-Turn/Multi-Function Lathe Machines saved within TechDB available for Selection in CAMWorks under Machine Tab of Machine Dialog Box

7 | P a g e

What’s New in CAMWorks 2026 BETA2

User-assigned names for Turrets / Tool posts associated with Mill-Turn / Swiss Lathes / Multi-Function Lathes

Purpose: To provide the option to assign user-assigned names for Turrets / Tool posts associated with Mill Turn/ Swiss Lathes/ Multi-Function Lathes so that they indicate the turret / tool post type (Gang Slide, Tool Post, etc.)

Implementation:

In the Technology Database, the Machines form present in Mill, Turn and Mill-Turn/Multi-Function  Lathe menus is used for viewing, creating, editing, and/or deleting virtual CNC machine definitions.  The Turret /Tool posts tab within the Machines form is used for viewing, defining, and editing  information associated with its tool cribs (usually turrets).

Renaming of Turret tab to Turret/Tool Posts Tab

From CAMWorks 2026 version onwards, within the Mill-Turn/Multi-Function Lathe> Machines form, the  Turret tab has been renamed to Turret/Tool Posts tab.

The renaming is deliberate as this specific tab can now be used for viewing, creating, or editing  turret / tool posts definitions for Swiss Lathe machines in addition to regular CNC Lathe machines.  While regular CNC Lathe/Mill-Turn machines support tool crib of type Turret, Swiss Lathes support  tool cribs such as Tool Posts and Gang Slides. The updated label of the tab serves as a visual  indicator of the expanded scope of this tab.

Turret/Tool Posts Tab under Mill-Turn/Multi-Function Lathe Machines Form within TechDB

Turrets vs. Gang Slides vs. Tool Posts

The table given below provides a basic summary of the features of these different tool crib types viz.  Turrets, Gang Slides and Tool Posts.

| Tool Crib Type Feature | Turret | Gang Slide | Tool Post |
| --- | --- | --- | --- |
| Motion Type | Rotary (indexing) | Linear (slides along X/Y/Z) | Static or on a slide |
| Tool Switching | Slower (requires indexing) | Faster (just moves slide) | N/A (Tool change by moving gang slide) |
| Tool Capacity | High (8–12+ stations) | Limited (depends on slide space) | Usually holds one tool |
| Precision | Moderate | Very high | High |
| Best For | Larger, complex parts | Small, high-precision parts | Holding tools (not a switching system) |
| Used In | Conventional CNC lathes | Swiss-type CNC lathes | Both |

8 | P a g e

What’s New in CAMWorks 2026 BETA2

Label Parameter under Turret/Tool Posts Tab

The first four collapsible group boxes under the Turret/Tool Posts tab are to define the number of tool  posts for the selected machine along with their associated properties. Each of these group boxes  represents a tool post that can be activated/deactivated as per the requirements for the specific  machine definition. In each of these group boxes, a parameter named Label is present. Use this  parameter to provide a short label for the specific turret / tool post.

It is highly recommended that you assign a label that provides information about the tool posts function/order/location/type. This is necessary as the label you assign will be displayed in the label  for the specific Tool Post. Observe the title header of the Turret / Tool posts group box (Rear 1, Front  1, Rear 2 and Front 2). The label you assign will be dynamically updated and displayed within  parentheses as a suffix to the title header of the Turret/Tool posts group boxes.  For example, if the name of the Turret / Tool post group box is Rear 1 and you assign the label as Gang  slide, then the Tool Crib group box name will be displayed as Rear 1 (Gang slide).

The Label assigned within a Tool Crib Group box will serve as the Name/Label for that Tool Crib (Applicable only for Tool Cribs assigned to Mill-Turn/Multi-Function Lathe/Swiss Lathe Machines)

Forms within TechDB where the User-Assigned Labels for Mill-Turn Turret / Tool posts will be Displayed The labels you assign to the Turret will also be displayed as column headers for the tables displayed  under group boxes labelled Station Index Times and Tool Changer Times under the Turret/ Tool Posts tab of the Mill-Turn/Multi-Function Lathe> Machines form.

User Interfaces within CAMWorks where the User-Assigned Labels for Mill-Turn Tool Cribs posts will be  Displayed

The labels you assign to the Turret / Tool posts will also be displayed within the following user  interfaces of CAMWorks:

• For all the turret / Tool posts listed in the Turret dropdown list under Tool Crib tab of the Machine dialog box

• For all the turret / tool posts listed in the Control Turret dropdown list under Step Through  toolpath dialog box

• For all the turret / tool posts listed in the Turret dropdown list under the Parameters tab of the  New Sub Spindle Operation dialog box when inserting a new Sub Spindle operation interactively • For all the turret / tool posts listed in the Turret dropdown list in the New Post Operation dialog  box

• For all the turret / tool posts listed in the Turret dropdown list of Tool Crib Page under Tool tab of  Operation Parameters dialog box

9 | P a g e

What’s New in CAMWorks 2026 BETA2

• For all the turret / tool posts listed in the Turret Selection dropdown list under the Tool tab of the  New Operation dialog box when inserting a new operation interactively

• For all the Turret nodes listed under the CAMWorks Tools tree

• As column headers of the Sync Manager Grid under Sync Manager tab of the CAMWorks Sync  Manager, where they indicate the names of the turret / tool posts

• As a suffix within parentheses for all the Operations nodes listed under the Operations tree (This  visually indicates the name of the turret / tool post the operation will be machined from.)

Tables under ‘Station Index Times’ and ‘Tool Changer Times’ group boxes displaying User-Assigned Turret  / Tool post Names as Column Headers

Turret Dropdown List displaying User-Assigned Turret / Tool post Names under Tool Crib tab of  Machine Dialog Box

10 | P a g e

What’s New in CAMWorks 2026 BETA2

User Assigned Turret / Tool post Names displayed User Assigned Tool Crib Names displayed  in Control Turret dropdown list under in Turret Dropdown List under Parameters tab  Step Through Toolpath Dialog Box of Sub Spindle Parameters Dialog Box

Turret Dropdown List displaying User-Assigned Turret / Tool post Names in New Post Operation  Dialog Box

11 | P a g e

What’s New in CAMWorks 2026 BETA2

Turret Dropdown List displaying User-Assigned Turret / Tool post Names under Tool Crib page of Tool Tab within Operation Parameters Dialog Box User Assigned Turret / Tool post Names displayed Turret / Tool post Dropdown List under for Tool Crib Nodes under Tools Tree Tool Tab of New Operation Dialog Box •

12 | P a g e

What’s New in CAMWorks 2026 BETA2

Machine Groups Option to sort and classify Mill-Turn/Multi-Function Lathe Machines

Purpose: To provide an option to sort the numerous Mill-Turn machines saved within the TechDB into different machine groups for enabling easier identification and classification

Implementation: The Technology Database allows you to define an unlimited number of Mill-Turn, Swiss Lathes and Multi-Function Lathe machines. To easily identify and classify such machines saved in the Mill Turn/Multi-Function Lathe>>Machines form of the TechDB, the option to create Machine groups and assign the saved machines to different Machine Groups has been introduced. ‘Machine Group’ Dropdown List under General Tab of Machines Form When a Mill-Turn, Swiss Lathe or Multi-Function Lathe machine available in the Mill-Turn/ Multi Function Lathe>Machines form is selected, its corresponding Machine Parameters form populated with the Machine Definition parameters will be displayed. Under the General tab of this Machine Parameters form, a new dropdown list labeled Machine Group has been introduced. All the Machine Groups saved within the TechDB are displayed within this dropdown list. Use this dropdown list to indicate the Machine Group that the selected Machine will be sorted into. Machine Group Dropdown List under General Tab of Machines Form in Mill-Turn/Multi-Function Lathe> Machines Form ‘None’ Option in ‘Machine Group’ Dropdown List under General Tab of Machines Form Assigning a Machine Group to a Mill-Turn machine type is optional. If you do not wish to assign the selected machine to any Machine Group, then select the None option in its Machine Group dropdown list. Sorting of Machines based on Machine Groups within the Mill-Turn/Multi-Function Lathe Interface In both the Mill-Turn/Multi-Function Lathe menu interface and Mill-Turn/Multi-Function Lathe>Machines form, all the machines will be sorted and listed based on their respective Machine Groups. All machines that have Machine Group assigned as None will be listed at the top. All other Machine Groups will be listed after the None machine group.

13 | P a g e

What’s New in CAMWorks 2026 BETA2

The machines under a specific Machine Group will be displayed in a collapsible group box. The Machine  Group will serve as the header of the collapsible group box. A numerical value will be displayed  within parentheses adjacent to the header. This number indicates the total number of machines  sorted into that specific machine group.

Creating, Editing and Deleting Machine Groups

Clicking on the Define Machine Group button adjacent to the Machine Group dropdown list in the  Machines form will display the Define Machine Group pop-up window. Use this window to view, edit, add, delete, and save machine groups in the TechDB. The machine groups you define will populate

the Machine Group dropdown list under General tab of the Machines form within the Mill-Turn/Multi Function Lathe>Machines form.

Machine Group Window to view, create, edit, and/or delete Machine Groups available in TechDB

Machine Groups under Machine Tab of Machine Dialog Box in the CAMWorks Application In the CAMWorks application, all machines of Mill-Turn type will be classified and listed in the  Available Machines list box under the Machine tab of the Machine dialog box on the basis of their  Machine Group.

Machines of Mill-Turn type classified on the basis of the Machine Group they belong to within  Available Machines list box under Machine Tab of Machine Dialog Box

14 | P a g e

What’s New in CAMWorks 2026 BETA2  Toolcrib Groups Option to sort and classify Mill-Turn Tool Cribs

Purpose: To provide an option to sort the numerous Mill-Turn machines saved within the TechDB into different machine groups for enabling easier identification and classification

Implementation: The Technology Database allows you to define an unlimited number of virtual Mill-Turn tool cribs. To easily identify and classify the numerous tool cribs saved in the Mill-Turn/Multi-Function Lathe>>Tool Cribs form of the TechDB, the following functionalities have been introduced: • The option to create Tool Crib groups • The option to assign a selected Mill-Turn tool crib saved in the TechDB to a Tool Crib group • For a selected Mill-Turn tool crib, the option to indicate the specific Machine Group for which it will be available (This will make the tool crib available for assignment to turrets/tool posts of any Mill-Turn/Multi-Function Lathe machine belonging to that Machine Group.) ‘Toolcrib Group’ Dropdown List in Pop-Up Window to Create/Edit Mill-Turn Tool Cribs In the Mill-Turn/Multi-Function Lathe>Tool Cribs form within the TechDB, clicking on the New or Edit button will display a pop-up window containing parameters associated with the specific tool crib's definition. These parameters are used to create or edit the selected tool crib. Pop-Up Window to Create/Edit Mill-Turn Tool Cribs The following new parameters have been introduced in this pop-up window: • Tool Crib Group Dropdown List: Use the Tool Crib Group dropdown list to assign the active tool crib to a specific Tool Crib Group available in this dropdown list. Assigning a Tool Crib Group to any Mill-Turn tool crib saved in the TechDB is optional. Hence, a Tool Crib Group named None is available in the Tool Crib Group dropdown list. If you do not wish to assign the active Tool crib to any Tool Crib group, then select the None option in the dropdown list. For all Mill-Turn tool cribs that are not sorted into any specific Tool Crib group, the default Tool Crib Group will be None. • Define Tool Crib Group Command Clicking on this command button will display the Define Tool Crib Group pop-up window. Use this pop-up window to create, edit, or delete Tool Crib Groups available within the TechDB. The Group Name column in this window indicates the names of all the Tool Crib Group as saved within the TechDB. A numerical value will be displayed within parentheses adjacent each tool crib group's name in this column. This value indicates the number of virtual Mill-Turn tool cribs saved in the TechDB that have been sorted into that specific Tool Crib group. The Tool Crib Groups defined and saved within this window will populate the ToolCrib Group dropdown list.

15 | P a g e

What’s New in CAMWorks 2026 BETA2

Tool Crib Group Pop-Up Window

• Available for ________ machine groups (Dropdown List)

Use this dropdown list to indicate the Machine group for which the active Mill-turn tool crib will  be available for selection when any machine belonging to that Machine Group is assigned as the  active machine.

If you wish to make the active tool crib available for all machines/machine groups, then select  the option All within this dropdown list.

Turret/Tool Posts Tab for Mill-Turn Machines

For Mill-Turn/Multi-Function Lathe machines, information associated with its tool cribs, turrets and/or  Tool Posts can be viewed, defined, and edited under the Turret/Tool Posts tab of the Mill-Turn/Multi Function Lathe>Machines form of the Technology Database. Use the Turret/ Tool Post dropdown list  present under this tab to select the default tool crib to be assigned for the selected turret of the  active machine. This dropdown list will be populated with tool cribs defined in the Tool Cribs form of  the TechDB. However, only tool cribs that have been assigned to the same Machine Group as the  active machine will be displayed in this dropdown list.

Turret/Tool Post Dropdown List under Turret/Tool Posts Tab of Machines Form in Mill-Turn/Multi Function Lathe> Machines Form

Viewing Tool Crib Groups on the Mill-Turn/Multi-Function Lathe Form

In the Mill-Turn/Multi-Function Lathe menu user interface, all the Mill-Turn tool cribs within the TechDB  are listed under the Tool Cribs sub-menu. In this user interface, all the virtual Tool Cribs saved  within the TechDB that are not sorted into Tool Crib group will be displayed at the top of the list box.  (These are the Tool Cribs that have the Tool Crib Group property assigned as None. After these non sorted tool cribs, the Tool Crib Groups will be listed in alphabetical order as headers of collapsible  groups. The tool cribs sorted into a specific Tool Crib group will be listed under that Tool Crib group  in the form of a collapsible box.

16 | P a g e

What’s New in CAMWorks 2026 BETA2

A numerical value will be displayed within parentheses adjacent to each header. This number  indicates the total number of tool cribs sorted into that specific Tool Crib group.

Mill-Turn Tool Cribs listed under Tool Cribs Groups they belong to in the Mill-Turn/Multi-Function  Lathe Menu User Interface

Available Mill-Turn Tool Cribs listed as per Tool Crib Groups in CAMWorks

In the CAMWorks application, when a Mill-Turn/Multi-Function Lathe machine is selected as the  active machine, the Available Tool Cribs list box under the Tool Crib tab of the Machine dialog box  will display and sort all the available Mill-Turn Tool Cribs on the basis of the Tool Crib Group assigned  to it. All the Mill-Turn Tool Cribs assigned a specific Tool Crib Group will be displayed under a label  bearing that Tool Crib Group’s name. All those Mill-Turn Tool Cribs that have not been assigned any  Tool Crib Group will be at the top of the Available Tool Cribs list box.

Available Tool Cribs List Box under Tool Crib Tab of Machine Dialog Box

17 | P a g e

What’s New in CAMWorks 2026 BETA2

Collets as Work Holding Device for Turn and Mill-Turn Machines

Purpose: To provide an option to define and assign a Collet as work holding device for the Main and/or Sub Spindles of Turn and Mill-Turn machines

Implementation: As a clamping device, collets are capable of producing a high clamping force and accurate alignment. In previous versions of CAMWorks, in Turn and Mill-Turn mode, only the Standard and user-defined chucks/fixtures were supported as work holding devices for Turn and Mill-Turn machines. From CAMWorks 2025 version onwards, an option to define a Collet as the work holding device has been provided. CAMWorks will allow users to define and maintain a centralized library of collets in Technology Database (TechDB). The user will be able to assign a desired collet saved in the TechDB to the Turn/Mill Turn machine selected for machining a part model/ assembly. Sample Collet Image Machine Dialog Box In the Machine dialog box, the Chuck/Fixture tab has been renamed to Fixture tab. The Shape dropdown list within the Main Spindle Information and Sub Spindle Information under this tab is used to determine the type of work holding device. A new option Collet has been added to this dropdown list. Once this option is selected, the name of the collet assigned to the machine will be displayed in the Name field below the Shape dropdown list. (This default selection is based on the default collet assigned to the Machine in the TechDB.) Collet option within the Shape dropdown list under Fixture tab of Machine dialog box

18 | P a g e

What’s New in CAMWorks 2026 BETA2

Collet Parameter Dialog Box

If you wish to assign another collet as the work holding device, then click on the Edit button  adjacent to the Name dropdown list under the Fixture tab. Clicking on this command button will  display the Collet Parameter: Main/Sub Spindle Dialog Box. Use the parameters and options available  within this dialog box to:

• Change the collet type/style and corresponding collet selection

• Create new collets and optionally save them to the TechDB

• Edit the parameters of the selected collet

Collet Parameter: Main/Sub Spindle dialog box displayed when Edit button is clicked

19 | P a g e

What’s New in CAMWorks 2026 BETA2

Collets Form in TechDB Within the TechDB, to invoke the Collets form, click on the Collets menu item under the Holders & Assemblies sub-menu of the Turn tooling menu. The Collets form allows you to create a centralized library of collets. All collets saved within the TechDB are displayed in a tabular format within the Collets form. Saving collets in TechDB using this form eliminates the need to define collets for individual Turn and Mill-Turn/Multi-Function Lathe machines. It thereby allows you to automate the selection of collets as a work holding device for Turn and Mill-Turn/Multi-Function Lathe machines in CAMWorks. Collets Form in Technology Database

20 | P a g e

What’s New in CAMWorks 2026 BETA2

Option to Define Collet Housing Parameters

Purpose: To provide the option to define basic parameters of a Collet Housing so that it can be visualized while programming a part

Implementation: 'Collet Housing' refers to the clamping fixture used to hold the collets. In the previous version of CAMWorks, no options were available to define a Collet Housing. From CAMWorks 2026 version onwards, a new group box labelled Collet Housing Parameters has been introduced within the Collet Parameters dialog box. Use the parameters available within this group box to define the collet housing parameters. Providing the definition for collet housing allows you to define and visualize the geometry of the collet housing directly within the graphics area. Sample of Collet Housing displayed along with the part in the Graphics Area Collet Housing Group Box within Collet Parameters Dialog Box The following parameters are made available in CAMWorks 2026 to define collet housing: • Collet Housing Diameter: Allows users to view, assign, or edit the largest diameter of the collet housing. • Collet Housing Length: Enables users to view, assign, or edit the overall length of the collet housing. • Minor Diameter: Users can view, assign, or edit the diameter of the front diameter section of the collet housing. • Collar Length: Allows users to view, assign, or edit the length of the front diameter section of the collet housing.

21 | P a g e

What’s New in CAMWorks 2026 BETA2

Collet Housing Parameters Group Box in Collet Parameters Dialog Box

Saving Default Collet Housing Parameters in TechDB

The parameters associated with defining Collet Housing are available in the Collets form provided  under Turn Tooling menu of the TechDB. You can assign the default values for Collet Housing definition within the TechDB using this form.

Collet Housing Parameters in Collets Form available under Turn Tooling Menu of TechDB

22 | P a g e

What’s New in CAMWorks 2026 BETA2  Activating Sliding Head Option for a Selected Machine

Purpose: To indicate that the active Mill-Turn/Multi-Function Lathe machine selected has a sliding head so that tools are assigned accordingly to all the operations and toolpaths are generated and post processed accordingly

Implementation: The Has sliding head checkbox option is present under the Fixture tab of the Machine dialog box when the active machine selected is a Mill-Turn/Multi-Function Lathe machines. Use this checkbox to indicate whether the machine is a traditional fixed head lathe machine or a sliding head lathe. When checked, it indicates that the active machine has a sliding head. ‘Has sliding head’ Checkbox Option under Fixture Tab of the Machine Dialog Box The default setting for this checkbox option can be assigned to the Has sliding head checkbox parameter present in the Spindle Tab under Mill-Turn/Multi-Function Lathe>Machines form in the Technology Database. Option to Assign Default Setting for ‘Has Sliding Head’ under Spindle Tab for a Mill-Turn/Multi Function Lathe Machine Defined and Saved within TechDB

23 | P a g e

What’s New in CAMWorks 2026 BETA2

Visualizing Tools in Graphics Area at Designated Distances For Machines with Sliding Heads While simulating the toolpaths on a Mill-Turn/Multi-Function Lathe Machine with a sliding head, the tools used for machining will be visualized at a designated location. Clicking on the Additional Parameters For Sliding Head command button adjacent to the Has sliding head checkbox will display the Additional Parameters For Sliding Head dialog box. Use this dialog box to assign/edit these default cutting locations (distances) as per the specifications required for the active machine. ‘Additional Parameters For Sliding Head’ Dialog Box • Zero Plane: Use this parameter to indicate the distance from the machine origin of the part to the face of the Guide Bushing. • OD Mill Tools: Use this parameter to indicate the distance from the machine origin to the centerline of Mill tools that will be used to machine the OD of the part. • Z Tools: Use this parameter to indicate the distance from the machine origin to the tip of mill and single point tools that will be used to machine the part from the Z direction.

24 | P a g e

What’s New in CAMWorks 2026 BETA2

Guide Bushings when Machining with Mill-Turn/Multi-Function Lathe Machines

Purpose: Support for Guide Bushings to enable machining with Swiss Turn/Multi-Function Lathe Machines

Implementation: What are Guide Bushings? Guide Bushings are a critical component of Swiss Machines and serves as a major factor differentiating them from conventional CNC Mill-Turn machines. They function as supporting devices for bar stock to be machined on Swiss Turn machines. A clamped bar stock can move axially through the guide bushings. Such a move ensures that the stock is rigidly supported at all times and the turning tool remains closer to the guide bushing. This enables the machining of parts with high quality finishes and extreme accuracies. Sample Image Displaying Guide Bushing During Simulation Guide Bushing Dialog Box in CAMWorks 2026 As Guide Bushings are integral to machining with Swiss Turn Lathes/Multi-Function Lathes with sliding heads, user interfaces for viewing, editing, adding, and assigning Guide Bushings have been introduced in CAMWorks. Enabling Guide Bushing within CAMWorks for a Selected Machine Under the Fixture tab of the Machine dialog box, a checkbox labeled Guide Bushing will be displayed when the following conditions are fulfilled: • The active machine is a Mill-Turn/Swiss Turn/Multi-Function Lathe machine with a sliding head • The Collet option is selected in the Shape dropdown list under the Fixture tab of the Machine dialog box • The Has sliding head checkbox option under the Fixture tab of the Machine dialog box is checked Place a check in the Guide Bushing checkbox if you wish to enable guide bushing for the selected collet. Name of Selected Guide Bushing The Name field adjacent to the Guide Bushing checkbox displays the name of the Guide Bushing currently used for the active machine. This is a read-only field. When the Guide Bushing checkbox is checked, the name of the first Guide Bushing saved in the library of Guide bushings within the TechDB will be displayed in this field.

25 | P a g e

What’s New in CAMWorks 2026 BETA2

Library of Guide Bushing Saved in Technology Database

Within the Technology Database, browse to the Turn Tooling menu. Clicking on the Guide Bushings  menu item under the Holders, Assemblies and Collets sub-menu will display the Guide Bushings form. Use the parameters available within this form to add, edit, copy, and/or delete Guide Bushings  to be used with your Swiss Machines/Multi-Function Lathe machines.

Guide Bushings User Inerface available under ‘Turn Tooling>Holder, Assemlies and Collets’ Menu of  Technology Database (TechDB)

Fixture Tab under Machine Dialog Box

Settings for Guide Busihing under Fixture Tab of the Machine Dialog Box

26 | P a g e

What’s New in CAMWorks 2026 BETA2

| Invoking the ‘Guide Bushing’ Dialog Box using Edit Button If you wish to change this default Guide Bushing that has been assigned, then click on the Edit... Button adjacent to this field. Use the displayed Guide Bushing dialog box to select another Guide Bushing or edit parameters of a selected Guide Bushing or to add and assign another Guide Bushing. The Available Guide Bushing dropdown list in this dialog box will be populated with Guide Bushings saved in the Technology Database. Once the Guide Bushing dialog box is closed and the interface reverts to the Fixture tab of Machine dialog box, the Name field will display the name of the newly selected Guide Bushing. | Guide Bushing Dialog Box |
| --- | --- |

27 | P a g e

What’s New in CAMWorks 2026 BETA2

’Initial collet rechuck Z’ to indicate distance between Collet and Part Origin

Purpose: Parameter to indicate the distance from the face of the collet to the origin of the part to be machined when a collet is used to clamp the workpiece/stock being machined on a Mill-Turn/Multi Function Lathe machine with sliding head

Implementation: From CAMWorks 2026 version onwards, when a Mill-Turn/Multi-Function Lathe machine with a sliding head is selected in the Machine dialog box, a new parameter Initial collet rechuck Z will be displayed under the Fixture tab of the Machine dialog box. The Initial collet rechuck Z parametric field will be enabled when the following conditions are fulfilled: • The active machine is a Mill-Turn/Multi-Function Lathe machine • The Collet option is selected in Shape dropdown list under Fixture tab of Machine dialog box. • The Has sliding head checkbox option under the Fixture tab of Machine dialog box is checked. The ‘Initial collet rechuck Z’ Parametric Field under Fixture Tab of Machine Dialog Box Use this parametric field to indicate the distance from the face of the collet to the origin of the part (Main Spindle Coordinate System). The value you assign to this parameter must be either zero or a value greater than zero. The distance value must be set such that the collet doesn't collide with the Guide Bushing during the machining of the part.

28 | P a g e

What’s New in CAMWorks 2026 BETA2

Image Illustrating indicating the ‘Initial collet rechuck Z’ distance when Machining a Part

Default Value of ‘Initial collet rechuck Z’

The default value for the Initial collet rechuck Z parameter can be assigned to the Initial collet  rechuck Z parameter under Spindle tab of the selected Mill-Turn/Multi-Function Lathe>Machines form within the Technology Database.

Initial collet rechuck Z parameter under Spindle Tab of Mill-Turn/Multi-Function Lathe>Machines Form of TechDB

How ‘Initial collet rechuck Z’ affects Simulation when Display of Machine Components is ‘ON’

If the Display Machine Components option is enabled, the length of the cylindrical stock displayed  for machining parts during simulation for Mill-Turn/Multi-Function Lathe machines with Sliding heads  will be computed as follows:

• Stock Length During Simulation = Initial collet rechuck Distance + Overall Length of Selected  Guide Bushing + Stock Length as defined in Stock Manager Dialog Box

• Stock Length During Simulation if Guide Bushing is absent = Initial collet rechuck Distance +  Stock Length as defined in Stock Manager Dialog Box

29 | P a g e

What’s New in CAMWorks 2026 BETA2  Support Turn And Mill-Turn Fixtures In Z Positive Space

Purpose: To allow the user to define the fixture of the main spindle in positive space of Z axis. Also, allow the coordinates of the toolpath to change as per the Z axis direction during step through and simulation.

Implementation: In previous versions of CAMWorks, the Z axis direction for Turn, Mill-Turn, Multi-function Lathe machines is set such that the Main Spindle is in the Z negative direction. The direction of the Main Spindle is critical as the posted coordinates for the toolpath during Step Through, Simulation and generation of NC code is determined by the Coordinate System. However, in some of the Swiss Turn machines, the positive space of Z axis will be towards the main spindle. From CAMWorks 2026 onwards, the option to set the main spindle in Z positive direction is introduced. This option is available to you in the form of a checkbox option. The checkbox option named Fixture in positive Z is added under the Main Spindle Coordinate System dialog box. ‘Fixture in positive Z’ checkbox option under the Main Spindle Coordinate System Dialog Box How the Checkbox option will work: • If the checkbox is unchecked, the Main Spindle will be in Z negative direction. (This default setting is defined in the Technology Database for the current machine.) • If the checkbox option is checked, the Main Spindle will be in Z positive direction. Depending on whether the checkbox option is checked or unchecked, the posted coordinates for the toolpath during Step Through, Simulation, and generation of NC code is determined and will be computed accordingly.

30 | P a g e

What’s New in CAMWorks 2026 BETA2

Main Spindle Fixture in Positive Z Direction after the ‘Fixture in Positive Z’ Checkbox Option is Checked When the Fixture in positive Z checkbox option is checked, multiple parameters associated with location or coordinates that are present in the following user interfaces will display toggled values. • Turn Part Setup Dialog Box o Origin Tab o Fixture Location Tab o Advanced Tab o Statistics Tab • Turn Operation Setup o Tool Tab o Tourn Rough Tab o Face Rough Tab o NC Tab o Advanced Tab o Statistics Tab • Segmentation Manager On the next page are some of the user interfaces that have parametric values getting affected when Main Spindle Coordinate System in Z positive direction:

31 | P a g e

What’s New in CAMWorks 2026 BETA2

Parameter Values under Origin tab of Operation Setup Parameters: [Main Spindle] dialog box when  Fixture in Positive Z checkbox is unchecked

Parameter Values under Origin tab of Operation Setup Parameters: [Main Spindle] dialog box when  Fixture in Positive Z checkbox is checked

32 | P a g e

What’s New in CAMWorks 2026 BETA2

Parameter Values under Tool tab of Operation Parameters dialog box when Fixture in Positive Z checkbox is unchecked Parameter Values under Tool tab of Operation Parameters dialog box when Fixture in Positive Z checkbox is checked When the fixture for the main spindle is set in positive Z space, the Z coordinates of the toolpath will be as per the assigned coordinate system. Correspondingly, the distances and/or locations of several vertices defined in the user interfaces for Feature parameters, Operation parameters, and Setup parameters will be displayed as per the Main Spindle Coordinate System. For example, any increment or decrement to clearance values will be as per the Main Spindle Coordinate System in Z positive direction.

33 | P a g e

What’s New in CAMWorks 2026 BETA2

Option to Enable/Disable the Creation of Duplicate Station Numbers for Stations in Tool  Cribs

Purpose: To provide an option to enable/disable the creation of duplicate station numbers for stations in tool crib(s) associated with the selected machine

Implementation: In previous versions of CAMWorks, the creation of duplicate station numbers for stations in a tool crib was allowed, but there was no option to disable this behavior. If a user wanted to prevent the creation of duplicate station numbers, no option was available to enforce it. From CAMWorks 2026 version onwards, to address this limitation, a new checkbox option labeled Allow Duplicate Station Numbers has been introduced under the Tool Crib tab of the Machine dialog box in the CAMWorks application. ‘Allow Duplicate Station Numbers’ checkbox under Tool Crib tab of Machines Dialog Box The Allow Duplicate Station Numbers checkbox enables users to control whether duplicate station numbers can be created in the tool cribs of the active part or assembly. • When this checkbox is checked, CAMWorks allows stations with duplicate station numbers, including substations in the tool cribs. • When this checkbox is unchecked, CAMWorks prevents the creation of duplicate station numbers for stations in the tool cribs. The default setting for this checkbox will depend on the setting assigned to the corresponding checkbox within the following TechDB forms: • Under the Turret tab of the Machines form under Mill menu of TechDB • Under the Turret tab of the Machines form under Turn menu of TechDB • Under the Turret/Tool Posts tab of the Machines form under Mill-Turn/Multi-Function Lathe menu of TechDB • • Note: For legacy parts (i.e., parts/assemblies programmed using CAMWorks 2025 or previous versions), the Allow Duplicate Station Numbers checkbox option will be in checked state by default.

34 | P a g e

What’s New in CAMWorks 2026 BETA2

‘Allow Duplicate Station Numbers’ checkbox in Turret tab of Mill > Machines Form in TechDB ‘Allow Duplicate Station Numbers’ checkbox in Turret tab of Turn > Machines Form in TechDB

35 | P a g e

What’s New in CAMWorks 2026 BETA2

Tools

Cutting Parameters for Turn Inserts Based on Stock Material

Purpose: To provide the ability to define cutting parameters for Turn Inserts based on the material of the stock being machined

Implementation: The Functionality of Defining Cutting Parameters for Tools In the previous version of CAMWorks, the functionality to define cutting parameters for Mill tools based on the stock material being machined is available. From CAMWorks 2026 version onwards, this functionality has been extended to Turn Inserts. For each Turn Insert, one or more Stock Material Groups can now be defined and cutting parameters can now be defined for Turn Inserts based on the stock material to be machined. Every Stock Material Group defined for a Turn Insert will have a set of cutting parameters associated with it that can be defined/edited in the Cutting Parameters dialog box (both in the Technology Database as well as the CAMWorks application). Under F/S tab of the Operation Parameters dialog box, a new option named Tool is now available in the Defined by dropdown list. When this Tool option is selected in the Defined by dropdown list, CAMWorks will use the feed and speed values defined in the Cutting Parameters dialog box. (The default setting for the Defined by dropdown list can be assigned in the Technology Database.) New ‘Tool’ Option introduced in the ‘Defined By’ Dropdown list under F/S tab New ‘Cutting Parameters’ Dialog Box in CAMWorks for Turn Inserts Cutting Parameters can now be defined for all types of Turn Inserts. However, these cutting parameters will be applied only when Tool option is selected in the Defined by dropdown list under F/S tab of the Operation Parameters dialog box for a Turn operation. A new Cutting Parameters button in the following user interfaces: • In the Turn Insert page under the Tool tab In Operation Parameters dialog box • Under the Turn Insert page in Edit Tool Parameters dialog box

36 | P a g e

What’s New in CAMWorks 2026 BETA2

Cutting Parameters button introduced in Turn Insert tab

Clicking on the Cutting Parameters button will display the Cutting Parameters dialog box.

Cutting Parameters Dialog Box

• Cutting Parameters when the Associate with Stock Material’ checkbox is checked

By default, the Associate with Stock Material checkbox option within this dialog box will be  checked. When checked, the Stock Material group will be automatically selected such that it is in  sync with the Stock Material defined in the Stock Manager dialog box. The cutting parameters  associated with the selected Stock Material Group will be populated will be populated within their  respective fields within the Cutting Parameters dialog box. These parameters associated with  Surface speed, Spindle speed and Feedrates will be disabled once populated.

37 | P a g e

What’s New in CAMWorks 2026 BETA2

• Cutting Parameters when the Associate with Stock Material’ checkbox is unchecked When unchecked, all the parameters associated with Surface speed, Spindle speed, Feedrates, and Coolant within this dialog box will be enabled. Select the desired Stock Material Group and then use these parameters to define/edit the cutting parameters. Defining Default ‘Cutting Parameters’ for Turn Inserts in Technology Database In the Technology Database, a new Cutting Parameters group box has been introduced in the Inserts form under Turn Tooling menu for all types of Turn Inserts. Use this form to define the default cutting parameter values that will populate the corresponding Cutting Parameters dialog box within the CAMWorks application when the Turn Insert is selected to machine a stock material. ‘Cutting Parameters’ Group Box introduced in Turn Inserts Form within Technology Database •

38 | P a g e

What’s New in CAMWorks 2026 BETA2

Option to Assign Tool Offset, Compensation IDs, and Tool Shift Parameters

Purpose: To provide the options to assign Tool Offsets, Compensation IDs, and Tool Shift Distance (distance between the face of the Guide Bush and the tools mounted on the station) to each station in the Tool Crib/Tool Post

Implementation: For Swiss-Turn machines and advanced Turn/Mill-Turn machines, a tool on an assigned station can have multiple offsets and Compensation IDs depending on the operation. Additionally, some Tool Cribs/Tool Posts on such machines do not have any movement along the Z axis. The tools mounted on such Tool Posts are set at a defined distance from the face of the Guide Bushing. An option to define the Tool Shift Distance (distance between the face of the Guide Bush and the tools mounted on the station) needs to be available. From CAMWorks 2026 onwards, CAMWorks allows assigning Tool Offsets, Compensation IDs, and Tool Shift Distance for each station. These can be used in the posted output to meet various machining requirements. A new dialog box named “Tool Offset and Comp ID Parameters” has been introduced to address these advanced Tool Crib/ Tool Posts settings. The Tools Offset and Comp ID Parameters dialog box appears when you click the Tool Offset and Comp ID Parameters command button below the Gage Offset parameters under the Station Page of the Tools tab in the Operation Parameters dialog box. The Tools Offset and Comp ID Parameters dialog box can be used to assign settings for the following parameters: • Primary Tool Offset Use this parameter to assign the primary offset for the selected station. Also, different Primary Offset values for different operations using the same tool assigned to a specific station applicable to the selected operation. • Compensation ID Use this parameter to assign the compensation ID for the selected station, applicable to the selected operation. • Secondary Tool Offset Use this parameter to assign the secondary offset for Groove Insert Tools. • Tool Shift Distance Use these parameters to define the distance between the face of the Guide Bush and the tool at the selected station. The new Tool Offsets and Comp ID Parameters dialog box

39 | P a g e

What’s New in CAMWorks 2026 BETA2

This new dialog box enhances flexibility and precision in tool offset and compensation management, thus ensuring optimal machining performance. Defining the default values for the Primary Offset, Secondary Offset, Comp ID, and Tool Shift distance in TechDB The default values for these parameters can be saved in the corresponding parametric fields of the Turn Tool Crib and Mill-Turn Tool Crib forms within TechDB. To assign default values for the tool stations in the TechDB, the option of assigning Primary Offset, Secondary Offset, Comp ID, and Tool Shift distance. Newly added Tool Offset, Comp ID, and Tool Shift parameters in the Tools form under Turn Tool Cribs.

based

40 | P a g e

What’s New in CAMWorks 2026 BETA2

Spindle Association with Tool Station to Define Work Area

Purpose: To allow users to define a particular spindle as the Work Area for each tool present in the active Tool Crib or added to the active Tool Crib so that each Tool station can machine only from the assigned spindle(s)

Implementation: For Turn operations, a new parameter Work Area (Spindle/s) will be available under the Station page of the Tool tab. Use this dropdown list to define the spindle(s) that will be allowed to use the selected Tool Station Number. The spindle options available in this dropdown list are: • Main • Sub • Both (Default option) Note: For all legacy parts programmed using a previous of CAMWorks, the Work Area (Spindle/s) property will be set to Both for all its Turn operations. Work Area (Spindle/s) Dropdown List introduced in Station Page under Tool Tab for Turn Operations How Work Area (Spindle/s) Property Affects the Active Tool Crib • Every tool present in the active Tool Crib will be assigned a Work Area (Spindle) • When a new tool is added to the active tool crib based on Tool Selection Conditions defined in the TechDB, then its Work Area (Spindle/s) property will be assigned as per the spindle on which the Turn feature is being machined.

41 | P a g e

What’s New in CAMWorks 2026 BETA2

How Work Area (Spindle/s) Property Affects Tool Assignment for Turn Operations on Turn/ Mill-Turn  Machines when the Generate Operation Plan Command is Executed

For Turn and Mill-Turn machines, every tool in its active Tool Crib is assigned a Work Area (Spindle/s) property under the Station page of the of that tool. For each Turn operation generated on executing  the Generate Operation Plan command, the Work Area (Spindle/s) will influence the tool assignment  for that operation as follows:

1. On Turn and Mill-Turn machines, when CAMWorks considers a tool present in the active Tool crib  for assignment to a Turn operation, it will check whether the Work Area (Spindle/s) property for  that tool is compatible with the Work Area (Spindle/s) property of that tool as saved in the TechDB. • If compatible, then the tool will be assigned to the operation.

• If not compatible, then a new tool with appropriate Work Area (Spindle/s) required for the  operation will be created and assigned to that operation. (This will work only if the 'Use  Tool Cribs Tools only' option under Tool Crib tab of Machine dialog box is unchecked.)

2. For any Turn operation generated on Turn or Mill-Turn machine, if the active Tool Crib does not  contain a tool that meets the selection criteria as defined in the TechDB for that operation, then  a new tool that fulfills the selection criteria needs to be added to the active tool crib. The Spindle  (Main/Sub) being used to machine the feature will determine the Work Area (Spindle/s) property for  this newly added tool.

How Work Area (Spindle/s) Property Affects Interactively Inserted Turn Operations when ‘Pick from Tool  Crib’ Option is selected

The New Operation dialog box is displayed when interactively inserting a Turn operation.  Under the Tool tab of this dialog box, when the Pick from the tool crib checkbox option is checked,  the Tool list box will display all eligible tools from the front and/or rear turrets that can be assigned  to the operation. However, only those Tools having a Work Area (Spindle/s) property compatible with  the Spindle on which the feature is being machined will be listed. (For example, if the feature is  being machined on the Main Spindle, then only tools having its Work Area (Spindle/s) property set to Main or Both will be listed in the Tool list box.

Tool List Box under Tool tab of New Operation Dialog Box

42 | P a g e

What’s New in CAMWorks 2026 BETA2

How Work Area (Spindle/s) Property Affects Interactively Inserted Turn Operations when ‘From Operation Parameter’ Option is selected Under the Tool tab of the New Operation dialog box, when the From operation parameter checkbox option is checked, you can use either the TechDB defaults or Copy from option available under the Operation tab of the New Operation dialog box to copy the parameters and tool assignment for the operation being added. • If the TechDB defaults option is selected, then the default tool assigned in the TechDB will be assigned to the operation and added to the active Tool Crib (if not already present). The newly added tool will be assigned a Work Area (Spindle/s) property valid for the spindle on which the feature is being machined. • When interactively inserting a Turn operation, if you select the Copy From option, then the tool of the operation selected for copying will be assigned to the current operation. If the Work Area (Spindle/s) property of the assigned tool is compatible with the spindle on which the feature is being machined, then the tool will be assigned. However, if the Work Area (Spindle/s) property for the assigned tool is not valid, the CAMWorks will change Work Area (Spindle/s) property to Both. CAMWorks will then display a warning message that states the following: "The Work Area (Spindle(s) of the tool station has changed. Select Add to create a new tool." Options for Add and Cancel will be available within the warning message box. Clicking on the Add option will close the warning message and add a new identical tool but with valid Work Area (Spindle/s) property to the active tool crib. Clicking on the Cancel option will close the warning message and the user interface will revert to the Operation tab of the New Operation dialog box. The Work Area (Spindle/s) property of the tool assigned to the operation selected for copying will be reverted to its original setting. Operation Tab of New Operation Dialog Box •

43 | P a g e

What’s New in CAMWorks 2026 BETA2

Option to Create Empty Tool Stations in Tool Cribs and associated Functionalities

Purpose: • To provide the option to define empty tool stations in a tool crib in CAMWorks • To provide an option to assign Station Type for Tool Stations in Mill-Turn Tool Cribs • To provide an option to remove a tool assigned to a Tool Station, leaving the tool station empty • To provide the option to create a new tool crib in TechDB with a user-specified number of empty tool stations that can later be populated • To provide an option that prevents new tool stations from being added to the selected tool crib • Addition of new tools to active tool crib when the Generate Operation Plan command is executed

Implementation: This functionality to create empty tool stations in tool cribs is especially beneficial for Swiss Turn machines that may have empty tool posts or turrets. From CAMWorks 2026 version onwards, you can define empty tool stations under the CAMWorks Tools Tree and also in Tool cribs within the TechDB. A context menu option to remove tools from tool stations of an active Tool crib under the CAMWorks Tools Tree has also been introduced. This feature is particularly useful for machine configurations where not all stations need to be occupied, ensuring flexibility and efficiency in tool management. Option to Define Empty Tool Stations in CAMWorks Under the CAMWorks Tools tree, a new context menu option named Add Station is now available. Executing this command will allow the addition of empty tool stations to the active Tool crib. The newly created empty station will be displayed as the last entry in the active Tool crib under the CAMWorks Tools Tree. ‘Add Station’ Option on Context Menu of Tool Node Under CAMWorks Tools Tree Tab Defining Empty Tool Stations in TechDB In the Tool Cribs form within the TechDB, an empty tool station can be added to the selected tool crib. Select the Empty Station option from the dropdown list displayed on clicking the Add button. The newly added empty tool station will be listed as the last entry in the selected Tool crib.

44 | P a g e

What’s New in CAMWorks 2026 BETA2  ‘Empty Station’ Option for Add Button in Tool Cribs Form within TechDB

Assigning Station Type for Tool Stations in Mill-Turn Tool Cribs

Every tool station within a Mill-Turn/Multi-Function Lathe tool crib will now a new property labelled  Station Type.

Within the TechDB, this property will be visible as the Station Type dropdown list is available in the  Tools form of the Mill-Turn/Multi-Function Lathe>Tool Cribs form.

Station Type dropdown List in Tools Form of Mill-Turn/Multi-Function Lathe>Tool Cribs Form

Use this Station Type parameter to assign the station type which will indicate the type of tool that can  be assigned to the specific station. The table given below indicates the tool types that can be  assigned to a tool station based on the station type assigned to it using the Station Type dropdown  list. These tool types will be displayed in the Type of Tool dropdown list in the Tools form of TechDB.

|  |  | Options available in the 'Type of Tool' dropdown list |
| --- | --- | --- |
| 1. | Station Type ‘Any’ | None, Insert-Holders, Mill Tools, Single Point Tools, User Defined Tools |
| 2. | Station Type ‘Turn’ | None, Insert-Holders, Drill, Center Drill |
| 3. | Live OD | None, Mill Tools, Single Point Tools, User Defined Tools |
| 4. | Live Face | None, Mill Tools, Single Point Tools, User Defined Tools |

45 | P a g e

What’s New in CAMWorks 2026 BETA2

Within the CAMWorks application, an empty Tool Station can be added to an active tool crib by  selecting the Add Station command in the context menu of a Tool node or Tool Crib node under the  Tools tree. The Edit Tool Parameters dialog box is displayed on executing this command. Under the  Station page of this dialog box, the Station type dropdown list will be displayed if the active machine  configuration is a Mill-Turn/Multi-Function Lathes. Within this dialog box, use this dropdown list to  indicate the station type for the new empty station being added to the active tool crib.

‘Station Type’ Dropdown list under Station Tab of ‘Edit Tool Parameters’ Dialog Box

Removing Tools from Tool Station Nodes

A new context menu option Remove Tool is now available for Tool nodes under the CAMWorks Tools  tree. It will be available in the context menu of only those tool nodes that are not in use by any of  the operations. Executing this command will remove the tool from the selected tool station node,  leaving it empty. The station node will be displayed as empty both under the CAMWorks Tools Tree as  well as the Tool Crib tab under Operation Parameters dialog box.

‘Remove Tool’ Option on Context Menu of Tool Node Under CAMWorks Tools Tree Tab

46 | P a g e

What’s New in CAMWorks 2026 BETA2

Removing Tools Assigned to Tool Station Nodes in TechDB

Within the Tool Cribs form of the TechDB, to remove a tool from a tool station of the active Tool crib,  select 'None' in the Type of Tool dropdown list within the Tools Form. This will set the Tool ID  parameter to -1, indicating successful tool removal.

‘None’ Option in Type of Tool dropdown list within Tools Form of Tool Cribs Form in TechDB

Creating a New Tool crib in TechDB with User-specified Number of Empty Tool Stations In Tool Cribs form of TechDB, a new checkbox option Create all stations as empty is available in pop-up  for creating new Tool Cribs. When this checkbox option is checked, all the tool stations in the newly  created tool crib will be empty. The number of empty tool stations created will be based on the  number assigned in the Tool Stations field within the pop-up.

‘Create all stations as empty’ Checkbox Option in Tool Cribs Form of TechDB

47 | P a g e

What’s New in CAMWorks 2026 BETA2

Option to prevent New Tool Stations from being added to selected Tool Crib

A new checkbox option Do not create new tool stations is now available under the Tool Crib tab of the  Machine dialog box. When checked, CAMWorks will not add a new tool station to the active Tool crib.  This option will be disabled when both the Tool Crib priority and Use toolcrib tools only checkbox options  are checked. The default for this checkbox can be defined in TechDB.

‘Do not create new tool stations’ checkbox Option under Tool Crib Tab of Machine Dialog Box

Option to prevent New Tool Stations from being added to selected Tool Crib in TechDB

Within the TechDB, the Do not create new tool stations checkbox option is available under the Turret tab of the Machines form. Use this option to assign the default setting for the corresponding  checkbox in the CAMWorks user interface (viz. Tool Crib tab under Machine dialog box).  ‘Do not create new tool stations’ Checkbox Option under Turret Tab of Machines Form

48 | P a g e

What’s New in CAMWorks 2026 BETA2

Addition of New Tools to Active Tool Crib when the ‘Generate Operation Plan’ Command is Executed When the Generate Operation Plan command is executed, new tools will be added to the tool crib from the tool library if they are required for machining the operations. • For Mill and Turn tool cribs, these new tools will be assigned to the empty tool stations provided they are compatible with the Tool Type property of that empty tool station. • For Mill-Turn tool cribs, these new tools will be assigned to the empty tool stations provided they are compatible with the Station Type and Tool Type property of that empty tool station.

49 | P a g e

What’s New in CAMWorks 2026 BETA2

Orientation for Mill Tools in Mill-Turn Tool Cribs

Purpose: To provide an option to define the Front or Back orientation for Mill Tools in Mill-Turn tool cribs

Implementation: In previous versions of CAMWorks, the option to define orientation for Mill Tools were Left, Right, Up, and Down. From CAMWorks 2026 version onwards, Mill-Turn/Multi-Function Lathe machines having sliding heads are supported. When parts/assemblies are machined on such machines, the tools can approach the spindle from the Y axis direction as well. Previous version of CAMWorks had no option available to assign the orientation for such tools. To address this limitation, for mill toolpaths machined on such machines having sliding heads, the Mill tool orientation can now be defined for tools approaching from the Y axis direction. Under the Mill Holder tab of a selected Mill tool, the following two newly added options will be available in the Orientation dropdown list: 1. Front 2. Back Note: The Front and Back options will be available in the Orientation dropdown list only if the active machine is a Mill-Turn/Swiss Machine/Multi-Function Lathe machine with a sliding head. Newly added ‘Front’ and ‘Back’ options in Orientation dropdown list under Mill holder tab for a Mill tool in a Mill-Turn Tool Crib

50 | P a g e

What’s New in CAMWorks 2026 BETA2

Factors affecting Orientation of Mill Tools assigned to Operations The table given below indicates the proper orientation that will be assigned to a Mill tool in the Mill Turn tool crib when the tool approaches from the Y axis direction: Spindle Turret Orientation for Feature Location on OD Main Spindle Rear Back Main Spindle Front Front Sub Spindle Rear Back Sub Spindle Front Front Assigning Default Orientation of Mill Tools present in Mill-Turn Tool Cribs saved in TechDB Use the Mill-Turn/Multi-Function Lathe >Tool Cribs form of the TechDB to assign the default Mill Holder orientation for tools in tool cribs to be assigned to Mill-Turn/Multi-Function Lathe machines. New ‘Front’ and ‘Back’ options under Mill Tool Orientation dropdown list in Tools Form of Mill Turn/Multi-Function Lathe>Tools Cribs Form in TechDB

51 | P a g e

What’s New in CAMWorks 2026 BETA2

New Tool Changer Option ‘Slide’ available For All Machine Configurations within TechDB

Purpose: To provide the option of new Changer method of Slide within Turret tab of Machine Configuration form within the Technology Database

Implementation: The Tool Changer for a CNC machine is a mechanical system that automatically switches cutting tools during the machining process. It enables these machines to perform multiple operations without manual intervention. In the Technology Database, the controls for the tool changer are available as the Changer dropdown list in the following forms of the Technology Database: • The Turret tab of the Mill Machines and Turn Machines forms within the TechDB • The Turret/Tool Posts tab of the Mill-Turn/Multi-Function Lathe> Machines form within the TechDB From CAMWorks 2026 version onwards, an additional option named Slide has been made available in this dropdown list. When this tool changer option is selected, the tools will be are mounted on a flat tool slide in a fixed arrangement. There will be no rotating turret. The machine will move the Gang slide to bring the desired tool into position. This tool changer method is ideal for Swiss type lathes and compact CNC machines that require faster tool changes. New ‘Slide’ option in ‘Changer’ Dropdown List under Turret/Tool Posts tab of the Mill-Turn/Multi Function Lathe>Machines Form in TechDB

52 | P a g e

What’s New in CAMWorks 2026 BETA2  Operations and Toolpaths

Segmentation Manager for Turn Operations

Purpose: To provide users with the option to split operations as per desired segment lengths and positions using a Segmentation Manager interface so that the part can be machined in multiple segments while being rigidly supported during the machining process

Implementation: The Need for Segmenting to Machine Turn Features In Swiss Turning, the machining length of any Turn feature has to be less than equal to the stroke length (i.e., Carbide Length in the Guide Bushing). If the feature being machined is longer than the Carbide Length, then the Turn feature needs to be machined in separate segments. The Segmentation Manager Wizard The newly introduced Segmentation Manager Wizard allows users to split Turn operations as per the segment lengths and positions. It is an interactive interface that allows users to perform segmentation for every Turn feature and operation as per their requirements. This wizard contains the Segments page and Operations page for defining segments and selecting operations for splitting, respectively. Segments Page of Operations Page of Segmentation Manager Wizard Segmentation Manager Wizard

53 | P a g e

What’s New in CAMWorks 2026 BETA2

Invoking the Segmentation Manager Wizard

The Segmentation Manager dialog box can be invoked by executing the following commands:

• By right-clicking on the CAMWorks NC Manager node in CAMWorks Operation tree and selecting  Segmentation Manager option in its context menu.

• By clicking on the Segmentation Manager command of the CAMWorks Command Manager.

Segmentation Manager Command on the Command Manager

Segments Page of the Segmentation Manager Wizard to Split Turn Operations

In the Segments page of the Segmentation Manager Wizard, every time the New button is clicked, a split  plane will be created and listed in the Split Planes list box. A corresponding segment will also be  created and listed in the Segments list box. The ‘segment’ indicates the portion of the feature that  will be machined by the operation. The split planes will be listed in the Split Planes list box in the  descending order based on their proximity to the front of the part. They will also be displayed in the  graphics area. Whenever you mouse hover over a split plane displayed in the graphics area, its  corresponding entry within the Split Planes list box will be highlighted.

The location of a split plane can be shifted by selecting a vertex/center point of an arc or circle along  the profile of the part model. The name of the selected entity will be displayed in the Entity field.

For a split plane selected in the Split Planes list box, use the Offset field to define an offset value for  the shifting it in Z direction. The offset distance will be measured from the assigned Entity.

• In the Turn Insert page under the Tool tab In Operation Parameters dialog box  • Under the Turn Insert page in Edit Tool Parameters dialog box

Sample Turn Part Model split into segments by adding Split Planes

54 | P a g e

What’s New in CAMWorks 2026 BETA2

Operations Page of the Segmentation Manager Wizard

In the Segments page of the Segmentation Manager Wizard, when you click on the Operations button  the Operations page of the Segmentation Manager Wizard will be displayed.

Operations Highlighted in Operations List Box within Operations Page

Within the Operations list box of the Operations page, all those operations that have their toolpaths  spanning across one or more segment boundaries will be highlighted. The highlighted operations are  the ones that are eligible for splitting into multiple operations based on the number of segments  that they span across. When you mouse hover over these highlighted operations, a tooltip stating  "Toolpath span is across ## segments" will be displayed. (The ## indicates the serial number of  segments that the toolpath intersects with.)

To split one or more desired operations from the highlighted operations, select those operations and  click on the Split Operations command button.

Split Operations Command in Operations Page of Segmentation Manager Wizard  The Split Operations command will be enabled when you select one or more highlighted operations  from the Operations list box. Executing this command will split each selected operation into multiple  operations based on the number of segments they span across. The number of split operations  created for each operation directly corresponds to the number of segments it spans across.  For example, if a Turn operation spans across three segments and the Split Operations command is  executed, then three split operations will be created.

Once this command is executed, each operation selected for splitting will be split as per the  dimensions of the segments it spans across. For each such operation, all its newly split operations  will be displayed as nested nodes under that operation in the Operations list box. When you highlight  such a nested operation, its corresponding toolpath will be displayed in the graphics area.

Nomenclature for Split Operations

The nomenclature syntax for these nested operation nodes indicating split operations will be as  follows:

<Operation Name><Segment #>[Tool Number - Turret Name]

Where:

• # indicate the segment number

• Turret name indicates the turret the tool is from (i.e., rear turret or front turret)

Update Operations Command in Operations Page of Segmentation Manager Wizard  The Update Operations command will be displayed in the Operations page when all the segment  dimensions are modified in the Segments page of the Segmentation Manager Wizard after one or more  operations listed in the Operations list box has been split. If a parent operation listed in the Operations list box has been split, then selecting that operation and executing the Update Operations command  will rebuild the toolpaths of the split operations as per the updated segment dimensions.

OK Button of Segmentation Manager Wizard

After the desired operations are split/updated, click on the OK button to save the segmentation data  and to implement these changes to the Operation tree.

When you click the OK button, CAMWorks will display the following message: “Do you want to sequence  the operations as per Z order?”

There will be two options within this message box viz. Yes and No.

• If you select Yes, the Segmentation Manager Wizard will close, and all the newly split operations  will be listed in the Operation tree as per the order of the segments. The toolpaths for this  newly split operations will be generated/rebuilt.

• If you select No, the Segmentation Manager Wizard will close and all the newly generated  operations will be listed in the Operation tree by replacing the original operation under the  same setup. The toolpaths for this newly split operations will be generated/rebuilt.

55 | P a g e

What’s New in CAMWorks 2026 BETA2

Operations supported for splitting using Segmentation Manager Wizard The following operations generated for OD Features are supported for segmentation within the Segmentation Manager Wizard: • Turn Rough • Turn Finish • Groove Rough • Groove Finish Operations supported for splitting using Segmentation Manager Wizard The following operations are not supported for segmentation within the Segmentation Manager Wizard: • Sub Spindle operations • Probe operations • Post operations • Face operations • Cut Off operations • Suppressed operations • Thread operations • All operations generated for ID features • VoluTurn operations (These are Turn Rough operations that have their Method set to VoluTurn) • Prime Turing operations (These are Turn Rough and Turn Finish operations that have their Method set to Prime Turning) Note: If the Segmentation Manager Wizard is invoked after splitting the operations, then all the split operations will be listed in the Operations list box. Any split operations that are deleted but still present in the Recycle Bin will also be listed in the Operations list box. However, deleted operations present in the Recycle Bin that were not split will not be listed in the Operations list box. •

56 | P a g e

What’s New in CAMWorks 2026 BETA2

New - Support for Bar Break Chamfering of the Stock in Turn Toolpaths

Purpose: To provide the option of adding Bar Break moves to Turn Toolpaths generated for OD features in order to prevent/eliminate burrs that can potentially damage the Guide Bushings

Implementation: During the Turn toolpath machining process, burrs (unwanted sharp edges) are formed by the tool inserts when machining the edges of the cylindrical stock. Burrs can cause potential damage when the stock material slides through Guide Bushings. From CAMWorks 2025 version onwards, to eliminate this issue of burr formation, the option to create additional moves called Bar Break moves has been provided for the following Turn toolpaths: - Turn Rough - Turn Finish - Groove Rough (Only for OD features) - Groove Finish (Only for OD features) - Face Rough - Face Finish Adding Bar Break moves to the cut passes ensures that the stock edges are deburred and Guide bushing doesn't get damaged when the stock moves in and out of the Guide Bushing during the machining process. Bar Break Parameters under NC tab of Operation Parameters Dialog Box Parameters associated with the Bar Breaks functionality has been provided under the NC tab of the Operation Parameters dialog box of all these above operations. Bar Break Parameters under NC Tab of Operation Parameters Dialog Box for Turn Rough Operation

57 | P a g e

What’s New in CAMWorks 2026 BETA2

Bar Break Options

• By default, the Bar Break Method under the NC tab will be set to None i.e., no Bar Break moves. • Select the Chamfer option in the Method dropdown list to add a chamfer to the sharp edges of  the stock along the OD feature. Use the Distance and Angle parameters within the Bar Break  group box to define the chamfer geometry.

• Select the Radius option in the Method dropdown list to add a fillet to the sharp edges of the  stock along the OD feature. Use the Distance and Radius parameters within the Bar Break group  box to define the fillet geometry.

Addition of Bar Break Moves to Turning Toolpaths

• For all the supported toolpaths, the Bar Break moves are added to the passes which intersect  the max stock diameter. These moves are appended intelligently to the regular cut moves as  and when needed.

| Sample Turn Rough Toolpath with Bar Break Chamfers added to First Cut and Final Cut Passes (The Bar Break moves are highlighted in a red rectangle.) | Sample Face Rough Toolpath with Chamfer Bar Break moves for each Cut Pass (The Bar Break moves are highlighted in the blue rectangle.) |
| --- | --- |

- For the Groove Rough and Finish toolpaths, the Bar Break moves are added intelligently  based on the groove style. The cutting pattern of the groove is considered for adding the  bar break moves. There can be only one or multiple Bar Break moves depending on the  groove style defined.

| Groove Rough Toolpath with ‘Zig’ Groove Style (Bar Break Move highlighted in blue circle) | Groove Rough Toolpath with ‘Zigzag’ Groove Style (Bar Break Move highlighted in blue circle) |
| --- | --- |

58 | P a g e

What’s New in CAMWorks 2026 BETA2

| Groove Rough ToolpathOrder: S123 Groove Style: Normal Groove Peck Type: None | Groove Rough ToolpathOrder: S321 Groove Style: Normal Groove Peck Type: None | Groove Rough ToolpathOrder: S213 Groove Style: Normal Groove Peck Type: None | Groove Rough ToolpathOrder: S231 Groove Style: Normal Groove Peck Type: None |
| --- | --- | --- | --- |
| Bar Break move added just before final cut pass. | Bar Break move added just before first cut pass. | Bar Break moves added before first cut and final cut passes. | Bar Break move added just before final cut pass. |

Reverse Option for Bar Breaks added to Turn Rough and Turn Finish Toolpaths

Use the Reverse option within the Bar Break group box to cut the Bar Break move in a reverse  fashion (i.e., the tool will approach the chamfer profile from max stock diameter and will machine  it). Placing a check in the Reverse checkbox option will machine the Bar Break in the direction  opposite to that of the cut passes.

The Reverse option is available only for Turn Rough and Turn Finish operations. However, this  checkbox option will be disabled if the Cut Type under the Turn Rough/Turn Finish tab is reversed.

| Sample Turn Rough Toolpath with Reversed Cut Type having Bar Break Chamfers added to First Cut and Final Cut Passes | Sample Turn Finish Toolpath with Reversed Cut Type having Chamfer Bar Break moves all Cut Passes |
| --- | --- |

59 | P a g e

What’s New in CAMWorks 2026 BETA2

Controlling Z Axis Movement for Toolpaths of Parts programmed for Swiss-Turn Machines

Purpose: For parts programmed to be machined Swiss Turn/Multi-Function Lathe machines, to assign Z Axis movement to either to the turret or the spindle of the machine depending on whether the tool assigned to an operation is positioned on a turret or Tool Post/Gang Slide

Implementation: Tool Positioning for Swiss-Turn/Multi-Function Lathe Machines In most Turn/Mill-Turn machines, the tools are positioned on turrets. In the case of Swiss Turn/Multi-Function Lathe machines, in addition to turrets, the tools can be positioned on Tool Posts and Gang Slides. Turret Guide Bush Main Spindle Collet Gang Slide Sub Spindle Tool Post Illustrative Image indicating the Tooling Related Components of a Swiss Turn Machine Z Axis Movement for Toolpaths of Parts programmed for Swiss-Turn/Multi-Function Lathe Machines Tools positioned on turrets can move in both X and Z axis. In contrast, only single-point tools can be positioned on Tool Posts and Gang Slides as these move only in either X axis or Y axis direction. For parts programmed to be machined on Swiss-Turn /Multi-Function Lathe machines, the Z axis movement for the toolpaths will be as follows: • For toolpaths that have been assigned tools positioned on Tool Posts/Gang Slides, the Z axis movement will be provided by the Main Spindle (i.e., the stock will move in Z axis direction). • For toolpaths that have been assigned tools positioned on turrets, the Z axis movement will be provided by the turret and the Main/Sub Spindle on which the stock is mounted will remain stationary. Controlling Z Axis Movement of Toolpaths within CAMWorks For each machining operation generated for a part to be machined on a Swiss Turn/Multi-Function Lathe machine, depending on whether the assigned tool is positioned on a turret or Tool Post/Gang Slide, the Z Axis movement needs to be assigned to either to the turret or the spindle of the machine. Within CAMWorks, parameters have been introduced within the Operation Parameters dialog box associated with operations generated for Swiss Turn/Multi-Function Lathe machines.

60 | P a g e

What’s New in CAMWorks 2026 BETA2

UI Parameters within CAMWorks to Control Z Axis Movement for Toolpaths generated for Swiss Turn  Machines

In CAMWorks, for operations that are to be machined on Swiss Turn/Multi-Function Lathe machines  with a sliding head, a new group box labelled Sliding Head has been made available in the Operation  Parameters dialog box. This group box and its parameters will be displayed within the dialog box only  if the following conditions are fulfilled:

i. The active machine is a Swiss Turn/Multi-Function Lathe machine with a sliding head (i.e., the Has Sliding Head checkbox under Fixture tab of the Machine dialog box is checked).

ii. The operation is machined on the Main Spindle.

Parameters within the ‘Sliding Head’ Group Box

• Z Axis Motion: This is a dropdown list containing two options. The default option selected will  be based on the option selected for this dropdown list parameter within the TechDB. The  following are the options within this dropdown list:

i. Move Stock: When this option is selected, the Z axis movement when machining the  toolpath will be provided by the stock.

ii. Move Tool: When this option is selected, the Z axis movement when machining the  toolpath will be provided by the tool assigned to the operation.

• Position the tool at: Use this parameter to indicate the absolute value in Z axis where the tool  will be positioned from the face of the Guide Bush. The value you assign must compulsorily be a  value greater than zero. The default value for this parameter will be the value defined for this  parameter within the TechDB.

Assigning Default Values within Technology Database for Parameters to Control Z Axis Motion

Given below is an image of the TechDB form for assigning Default Operation Parameters for Turn  operations. The default Sliding Head parameters are to be assigned under NC tab of the selected  operation.

TechDB Form for Assigning Default Operation Parameters for Turn Operations

61 | P a g e

What’s New in CAMWorks 2026 BETA2

UI Parameters to Control Z Axis Movement for Turn Toolpaths

For Turn operations, this Sliding Head group box containing parameters to control the Z axis  movement is available under NC tab of the Operation Parameters dialog box.

Sliding Head Group Box under NC Tab for Turn Operations

UI Parameters to Control Z Axis Movement for 2.5 Axis/3 Axis Mill Toolpaths

For 2.5 Axis and 3 Axis Mill operations, this Sliding Head group box containing parameters to control  the Z axis movement is available under NC tab of the Operation Parameters dialog box.

Sliding Head Group Box under NC Tab for 2.5 Axis Mill Operations

62 | P a g e

What’s New in CAMWorks 2026 BETA2

Sliding Head Group Box under NC Tab for 3 Axis Mill Operations

UI Parameters to Control Z Axis Movement for Multiaxis Mill Toolpaths

For Multiaxis Mill operations, this Sliding Head group box containing parameters to control the Z axis  movement is available under Entry Retract tab of the Operation Parameters dialog box. Sliding Head Group Box under Entry/Retract Tab for Multiaxis Mill Operations

63 | P a g e

What’s New in CAMWorks 2026 BETA2

Indicating whether a Tool Post/ Turret/ Gang Slide of Swiss-Turn Machines is capable of  movement in Z axis

Purpose: Option to indicate whether a Tool Post/ Turret/ Gang Slide is capable of movement in Z axis or not

Implementation: For a Mill-Turn/Multi-Function Lathe machine with Sliding Head, the tools loaded on some of the Tool Posts/ Turrets/ Gang slides can move only in X axis or Y axis direction. Such Tool Posts/ Turrets/ Gang Slides of such machines do not have movement in Z axis direction. It is necessary to indicate whether the tool or stock will provide the Z axis movement for the toolpath to ensure error free machining. From CAMWorks 2026 version onwards, a checkbox option labelled Moves in Z Axis has been introduced. In the TechDB, this option is available for all Turret/Tool posts under the Turret/Tool posts tab of the Mill-Turn/Multi-Function Lathe > Machines form. This checkbox is unchecked by default. Use this checkbox option to specify if the Turret/Tool Post has movement in Z Axis or not. • If this checkbox option is unchecked, it indicates that the selected Turret/Tool Post has no movement in Z axis. The Z axis movement when machining the toolpath will be provided by the stock. • If this checkbox option is checked, it indicates that the selected Turret/Tool Post has movement in Z axis. The Z axis movement when machining the toolpath will be provided by the tool assigned to that operation. (The tool is loaded on the selected Turret/Tool Post.) ‘Moves in Z axis’ checkbox option under the Turret/Tool posts tab of the Mill-Turn/Multi-Function Lathe > Machines form Within the CAMWorks application, the Moves in Z Axis checkbox option will be displayed under the Tool Crib tab of Machine dialog box as a read-only field. This checkbox will be displayed only if the active machine is a Mill-Turn/Multi-Function Lathe machine with a sliding head (i.e., the Has Sliding Head checkbox under Fixture tab of the Machine dialog box is checked). Its checked/unchecked status will be determined by the selection made for corresponding Moves in Z Axis checkbox under Turret/Tool Posts tab under the Mill-Turn/Multi Function Lathe>Machines form within the TechDB.

64 | P a g e

What’s New in CAMWorks 2026 BETA2

‘Moves in Z axis’ checkbox option under the Tool Crib tab of Machine dialog box as a read-only field

65 | P a g e

What’s New in CAMWorks 2026 BETA2

Associating Sub Spindle operations with other Operations to ensure Auto-Synchronization

| Purpose: To enable users to associate Sub Spindle operations with any other operation, ensure automatic synchronization of these operations within the Sync Manager, and generate the appropriate sub spindle support codes in the posted output. |  |
| --- | --- |
| Implementation: In previous versions of CAMWorks, Sub Spindle operations could not be synchronized with cutting operations. New Parameters in Sub Spindle Parameters Dialog Box From CAMWorks 2026 version onwards, the option to associate Sub spindle operations with other operations has been introduced. The Sub Spindle Parameters dialog box is displayed on executing the New Sub Spindle Operation command. Under the Parameters tab of this dialog box, a new checkbox option Associate to an operation has been introduced. Place a check in this checkbox option if you wish to associate the Sub Spindle operation with any of the other operations machined from the Main Spindle. Placing a check in this checkbox will enable the Associate to dropdown list. Use this dropdown list to select the operation to associate the Sub Spindle operation with. (This dropdown list will list all the operations under the selected Turn Setup with the exception of Probe operations, Post operations, and other Sub Spindle operations.) Advantages of Associating Sub Spindle Operation with other Operations Associating a Sub Spindle operation with another operation ensures that both these operations are automatically synchronized in the CAMWorks Sync Manager. This functionality is especially useful for synchronizing Cut Off operations with Sub Spindle operations when machining using Turn, Swiss Turn, Mill-Turn or Multi-Function Lathe machines. | New Parameters in the Sub Spindle Parameters UI Default Selection in ‘Associate To’ dropdown list: The Sub Spindle Operation dialog box is invoked by executing the New Sub Spindle Operation command. The location this command is executed from determines the default selection in the Associate To dropdown list. • If invoked from the Command Manager, then the last operation listed under the Turn Setup node in the Operation tree will be listed as the default operation in this dropdown list. • If invoked from the context menu of the Turn Setup node in the Operation tree, then the last operation listed under that Turn Setup will be listed as the default operation in this dropdown list. • If invoked from the context menu of any operation node in the Operation tree, then that operation will be listed as the default operation in this dropdown list. |

66 | P a g e

What’s New in CAMWorks 2026 BETA2  Chip Breaking Functionality for Turn Operations

Purpose: To provide the option to users to enable the Chip Breaking functionality for Turn operations so that the chips generated during the machining process are short

Implementation: During the Turn machining process, the machining of metals may result in long chips which can potentially reduce efficiency and tool life. The newly introduced Chip Breaking functionality within the CAMWorks application provides a solution by breaking these chips to smaller, manageable pieces. Enabling the Chip Breaking functionality and assigning values to parameters associated with this functionality will modify the resulting toolpath is such a way that the only short chips are formed. Turn Operations for which Chip Breaking Functionality is Available The option to enable Chip Breaking is available for the following Turn operations: • Turn Rough operations (within the Cut Type group box under the Turn Rough tab) • Turn Finish operations (within the Cut Type group box under the Turn Finish tab) • Bore Rough operations (within the Cut Type group box under the Bore Rough tab) • Bore Finish operations (within the Cut Type group box under the Bore Finish tab) • Face Rough operations (within the Cut Type group box under the Face Rough tab) • Face Finish operations (within the Cut Type group box under the Face Finish tab) Invoking the Chip Breaking Dialog Box In the above-listed tab interfaces, within the Cut Type group box, placing a check in checkbox adjacent to the Chip Breaking push button will activate Chip Breaking push button. Clicking on this push button will invoke the Chip Breaking dialog box. Note: • For Turn Rough, Face Rough, and Face Rough operations, the checkbox adjacent to the Chip Breaking push button will be enabled only if the Canned Cycle Output checkbox option is unchecked. • The Chip Breaking functionality is not available for Turn Rough-VoluTurn and Turn Rough- Prime Turning operations. Parameters in the Chip Breaking Dialog Box Chip Breaking Dialog Box

67 | P a g e

What’s New in CAMWorks 2026 BETA2

Length of Cut: Use this parameter to indicate the distance along the cut at which a chip breaking point will be induced in the toolpath i.e. the distance between consecutive chip breaking points in the toolpath. The value you assign to this parameter must be greater than zero. The default value for this parameter will be picked from the TechDB. In the default TechDB shipped with the CAMWorks application, this default value will be 25mm (1 inch). Retract Distance Use this parameter to indicate the distance the tool must traverse in the reverse direction from the chip break point. The value can be 0 or more. The default value will be zero or as set in the TechDB. Angle When Chip breaking is enabled for a toolpath, the tool traverses in the reverse direction from the chip break point based on the value assigned to the Retract distance parameter. For this retract move, the angle will be measured with respect to the segment of the toolpath in which the Chip breaking point is located. The default value will be 45 degrees or as set in the TechDB. The allowed value range for this parameter is from 0 degrees to 90 degrees. Dwell • When the None option is selected for this parameter, the tool will not dwell after traversing the retract distance. It will immediately restart the cutting moves as per the toolpath. • When the Time option is selected, the tool will dwell after traversing the retract distance. Use the parametric field adjacent to this option to indicate the dwell time in seconds. The tool will remain at the end of the retract distance for the time duration mentioned in this field. The default value for this parameter will be picked from the TechDB. Assigning Default Values for Parameters associated with Chip Breaking functionality in TechDB In the corresponding Operation Forms for supported Turn operations, placing a check in the Chip Breaking checkbox will activate parameters associated with the Chip Breaking functionality. Use these parameters to assign/edit these parameters. Chip Breaking Parameters in Technology Database

68 | P a g e

What’s New in CAMWorks 2026 BETA2

Simulation and Visualization

Display of Machine Kinematics during Toolpath Simulation

Purpose: Support for displaying Machine Kinematics during Toolpath Simulation

Implementation: In the graphics area of CAMWorks, the commands available within the Display Components toolbar are used to enable/disable the display of the various groups of machine components comprising the Simulation Machine. From CAMWorks 2026 version onwards, if the commands available within the Display Components toolbar are enabled to display machine components, then on executing the Simulate Toolpath command, Machine Kinematics will be observed in the graphics area during the entire simulation process. Mill Mode In Mill mode, Machine Kinematics simulation is supported for the following Mill Machine types: • 3 Axis Mill Machines • 4 Axis Mill Machines • Horizontal 4 Axis Mill Machines Machine Kinematics in Graphics Area during Toolpath Simulation for a Horizontal 4 Axis Mill Machine Turn Mode In Turn mode, Machine Kinematics simulation is supported for the following Turn Machine types: • Single Turret Turn Machines (2 Axis Turn) • Dual Turret Turn Machines (4 Axis Turn) • Multi Turret Turn Machines

69 | P a g e

What’s New in CAMWorks 2026 BETA2

Machine Kinematics in Graphics Area during Toolpath Simulation for a Turn Machine

Mill-Turn Mode

In Mill-Turn mode, Machine Kinematics simulation is supported for the following Machine types: • Single Turret Mill-Turn Machines

• Dual Turret Mill-Turn Machines

• Multi Turret Mill-Turn Machines

• 1H_1T Mill-Turn Machines (with one Turret and one Mill tool crib)

Machine Kinematics in Graphics Area during Toolpath Simulation for a 1H 1T Mill-Turn Machine

70 | P a g e

What’s New in CAMWorks 2026 BETA2

Machine Awareness Functionality Support For CNC Machines (Including Swiss Lathes)

Purpose: To ensure Machine Awareness functionality is supported for all CNC machines within the CAMWorks application

Implementation: Machine Awareness refers to CAMWorks’ ability to interpret, display, replicate and simulate the physical environment of the selected CNC machine in the graphics area. CAMWorks achieves Machine Awareness by creating a digital representation of the selected CNC machine, inclusive of all its components, to allow for a more realistic simulation. Pre-requisite for Enabling Machine Awareness Functionality • To access the Machine Awareness functionality in CAMWorks, your CAMWorks license must be configured to run this functionality. Contact your reseller if you wish to get this functionality activated. • Once you license is configured to execute the Machine Awareness functionality, the Simulation Machine dropdown list under Machine tab of the Machine dialog box will display a list of all the virtual simulation machines that can be simulated. Use this dropdown list to select the machine to be simulated. Selecting a Simulation Machine in the Simulation Machine dropdown list is a pre requisite to enable Machine Awareness for the specific part/assembly being machined. Simulation Machine Dropdown list under Machine tab of Machine Dialog Box

71 | P a g e

What’s New in CAMWorks 2026 BETA2

Advantages of Using ‘Machine Awareness' Functionality

• Graphical Awareness: Machine awareness provides CAM programmers the ability to create a CNC  setup and program a part or assembly on a digital twin of the CNC machine. As CAMWorks is  seamlessly integrated with SOLIDWORKS/CAMWorks Solids, this digital twin of the selected CNC  machine can be viewed in the graphics area along with the part/assembly being machined. By  including a digital twin of the CNC machine during the programming workflow, CAM programmers  are provided with instant graphic awareness of the machine’s table size, travel limits, rotational limits, and available tooling. This helps in ensuring the viability of the planned machining process.

• Improved Toolpath verification: Machine awareness helps in understanding the selected CNC  machine's capabilities and limitations. This helps programmers to generate more efficient and  reliable toolpaths.

From CAMWorks 2026 version onwards, the Machine Awareness functionality has been further enhanced  to ensure that the display and simulation of the machine components is rendered more realistically vis-à-vis the previous versions.

Activating ‘Machine Awareness’ using the ‘Display Components’ Toolbar

The enabling/disabling of the display of various machine components (i.e., Machine Awareness) is  controlled through the Display Components toolbar.

The Display Components toolbar will be displayed in the graphics area of the CAMWorks user  interface in the following scenarios:

• When you execute the Step Through Toolpath command

• When you execute the Simulate Toolpath command

• When you click on the Display Components command available within the CAMWorks Command  Manager

‘Display Components’ Command in CAMWorks Command Manager

Display Machine Components Toolbar as displayed in the Graphics Area

Display Component toolbar Commands enabled During Toolpath Simulation

The following command buttons and options of the Display Components toolbar will be enabled  during the Toolpath Simulation process:

• Display Machine: Use this toggle button to enable/disable the display of machine  components of the Simulation Machine in the graphics area.

• Display Housing: Use this toggle button to enable/disable the display of the 'machine  housing' in which the machine components are installed. This toggle

button will be disabled in the toolbar during execution of the Step

Through toolpath and Simulate Toolpath commands.)

• Virtual Machine Stock Mounting: This button will be available in the Display Components  toolbar only for Mill machines. This button will be

disabled in the toolbar during execution of the Step

Through toolpath and Simulate Toolpath commands.)

When this toggle button is set to 'ON' status, the dialog

box to define the virtual machine stock mounting point

will be displayed.

72 | P a g e

What’s New in CAMWorks 2026 BETA2

• Display Turret: This toggle button is displayed within the toolbar only when the  selected Simulation machine is a Turn or Mill-Turn/Multi-Function

Lathe machine. Use this toggle button to enable/disable the

display of the turret(s) and tools loaded on those turrets.

• Tool & Tool Holder Display: Use this button to change the display of the machining tools and  holders in the graphics area.

• All Tools (Checkbox): This checkbox is displayed within the toolbar only when the  selected Simulation machine is a Turn or Mill-Turn/Multi-Function

Lathe machine. Use this checkbox option to enable/ disable the

display of tools loaded on the turret in the graphics area.

Sample Toolpath Simulation when Simulation Machine is a Multi-Function Lathe Machine

Machine Awareness Functionality for Mill-Turn/ Multi-Function Lathes/ Swiss Lathes

The Machine Awareness functionality will be supported for the newly introduced Swiss Turn machines.  To render Machine Awareness for a Swiss Turn machine, ensure that the Simulation Machine template  file and Controller emulation template file are selected under Machine tab of the Machine dialog box  when programming the part/assembly.

Note that Machine Awareness functionality is not supported for EWC (Encrypted Work Cell) based  machines (i.e., encrypted machines). If the Simulation Machine selected is such an encrypted  machine, then all commands within the Display Components toolbar will be disabled.

Display Machine Components Toolbar with all Commands Disabled

73 | P a g e

What’s New in CAMWorks 2026 BETA2

Illustrative Image displaying Machine Awareness for a Swiss Turn Machine  Select the desired Swiss Turn Template and Controller under Machine tab of Machine Dialog Box

74 | P a g e

What’s New in CAMWorks 2026 BETA2

Support for Swiss Turn Machines in CAMWorks Virtual Machine Environment

Purpose: Support for simulation of programmed parts/assemblies to be machined using Swiss Turn machines within the CAMWorks Virtual Machine application

Implementation: The CAMWorks Virtual Machine is the resident Machine Simulation application available as part of the CAMWorks suite of licensed modules. G-codes generated for parts/assemblies programmed using the CAMWorks application can be simulated in the CAMWorks Virtual Machine application. To use this application, your CAMWorks license must be configured to run this application. From CAMWorks 2026 version onwards, parts/assemblies can be programmed to be machined on Swiss Turn machines. The G-code generated for such parts/assemblies can be simulated in the CAMWorks Virtual Machine application. The CAMWorks Virtual Machine application will have a pre-defined library of Machine templates and CNC controllers for all demo Swiss Turn machines shipped with the CAMWorks application. If you wish to simulate a custom Swiss Turn machine within the CAMWorks Virtual Machine environment, then ensure that you procure its corresponding Simulation Machine template file and Controller emulation template file from your reseller. Ensure that these template files are selected under Machine tab of the Machine dialog box when programming the part/assembly. Select Desired Simulation Machine and Controller under Machine Tab of Machine Dialog box

75 | P a g e

What’s New in CAMWorks 2026 BETA2

Image Demonstrating Swiss Turn Machine Simulation within the CAMWorks Virtual Machine Environment

76 | P a g e

What’s New in CAMWorks 2026 BETA2

Expanded Sync Manager View for CAMWorks Sync Manager

Purpose: To provide the option for an expanded view of synchronized operations machined on different spindles (Main Spindle/ Sub Spindle) using tools from different turrets/tool posts/tool cribs within the CAMWorks Sync Manager dialog box

Implementation: In previous versions of CAMWorks, for Turn and Mill-Turn machines, the CAMWorks Sync Manager functionality allows you to synchronize the toolpaths machined by the tools in two or more turrets/tool cribs/tool posts. This is achieved by re-ordering the operations in the CAMWorks Sync Manager user interface. Optimal synchronization can drastically reduce the time required to machine the parts/assemblies. From CAMWorks 2026 version onwards, the CAMWorks Sync Manager functionality has been enhanced to provide an expanded view of the synchronized operations. If the Syncing Strategy is set to User Defined and the Expand checkbox within the Sync Manager View group box is checked with the By Turret or By Spindle option selected, then an expanded view of the Sync Manager grid based on the Turret-Spindle or Spindle-Turret combinations will be displayed. ‘By Turret’ Option for Expanded Sync Manager View If the By Turret option is selected, the Sync Manager grid will have a column header indicating the Turret-Spindle combination info. - The first part of the column header’s name will be the name of the turret. A descriptive label within parentheses suffixed to the turret name will indicate its function/location/order/type. - The second part of the column header’s name will indicate the spindle on which operations will be machined, viz. 'Main Spindle' and 'Sub Spindle'. - Each column will list only those operations that have tools assigned from that specific turret and are machined from the specific spindle as indicated by the corresponding column name. - These operations will be listed as per their machining sequence within each column. Expanded Sync Manager View when ‘By Turret’ Option is Selected

77 | P a g e

What’s New in CAMWorks 2026 BETA2

‘By Spindle’ Option for Expanded Sync Manager View If the By Spindle option is selected, the Sync Manager grid will have two-tier column headers that indicate the Turret-Spindle combination info. - The first-level column header will be the name of the Spindle (viz. 'Main Spindle' and 'Sub Spindle'). - Each first-level header indicating the Spindle will span across one or more second-level column headers that indicate the name of the turret. A descriptive label within parentheses suffixed to the turret names in the second-level column headers will indicate their function/location/order/type. - Each column in the grid will list only the operations that are machined from that specific spindle (first-level column header) and have tools assigned from that specific turret (second-level column header). These operations will be listed as per their machining sequence within each column. Expanded Sync Manager View when ‘By Spindle’ Option is Selected

78 | P a g e

What’s New in CAMWorks 2026 BETA2

Providing Visual Feedback on Start Location of Wait Codes within CAMWorks Sync Manager

Purpose: To provide the visual feedback (in the form of virtual operations) about the start location of the wait codes for synchronized operations within the Sync Manager grid of the CAMWorks Sync Manager dialog box to indicate whether the wait codes is positioned after a tool change or after the first feed move

Implementation: Within the Sync Manager tab under the CAMWorks Sync Manager dialog box, wait codes can be added to operations machined from different turrets/tool posts/tool cribs to sync them with other operations. Using the Wait Code commands available within this tab, wait codes can be added at the following locations: - Before the start of the next operation - Before the first rapid move of the next operation - Before the first feed move of the next operation For a wait code already inserted, its start location can be edited by double-clicking on the wait code to invoke the Wait Code dialog box. Use this dialog box to Wait Code Properties Dialog Box In previous versions of CAMWorks, all inserted wait codes were visible within the Sync Manager grid under Sync Manager tab. However, there was no visual info on their start locations i.e., whether the wait code was inserted before the start of the next operation or before the first rapid move or feed move of the next operation.

79 | P a g e

What’s New in CAMWorks 2026 BETA2

From CAMWorks 2026 version onwards, a mechanism to provide visual indication about the start  location of the inserted wait codes within the Sync Manager grid under Sync Manager tab has been  introduced. This visual feedback is provided in the form of virtual operations that indicate the start  location of the wait code.

Start Location Indicator for Wait Code when Wait Code Property is set to 'Before the Start of the next  operation'

If the wait code property indicating the start position is set to 'Before the Start of the next operation',  then no entry indicating the start location of that wait code will be displayed in the Sync Manager grid  under the Sync Manager tab.

The wait code will be located immediately above the operation in the column of the Sync Manager  grid.

Wait Code when Start Location is ‘Before the Start of the next operation’

Start Location Indicator for Wait Code when Wait Code Property is set to 'Before the First Rapid the next  operation'

If the wait code property indicating the start position is set to Before the First Rapid of next operation,  then the wait code will be positioned after the tool change move of the operation before which the  wait code has been inserted. An entry indicating the start location of the wait code will be displayed  in the Sync Manager grid under the Sync Manager tab.

The syntax for the entry indicating the start position of the wait code will be: <Name of the operation before which the wait code was inserted > - Tool Change [<Tool Station Number>]

Start Location Indicator for Wait Code when Start Location is ‘Before the First Rapid of next operation’

Start Location Indicator for Wait Code when Wait Code Property is set to 'Before the First Feed of next  operation'

If the wait code property indicating the start position is set to Before the First Feed of next operation',  then the wait code will be positioned after the first non-cutting move of the operation before which  the wait code has been inserted. An entry indicating the start location of the wait code will be  displayed in the Sync Manager grid under the Sync Manager tab.

The syntax for the entry indicating the start position of the wait code will be: <Name of the operation before which the wait code was inserted > - First Move [<Tool Station Number>]

80 | P a g e